SysMetEx - Standard Operating Procedures

RNA/Protein Samples (LNU)

Materials:

- 50 ml Falcon tubes

- 2 ml RNAse-free microcentrifuge tubes

- Ice cold Mackinthosh medium (MAC)

- Liquid Nitrogen

Procedure:

- Prepare 4x 50 ml Falcon tube containing 25 ml ice cold MAC

- Add 25 ml culture/reactor content to the tubes -> 4x makes 100 ml sample total

- Spin down at max speed, 4 °C, 15 min, slow deceleration

- Decant supernatant

- Resuspend pellet in 10 ml ice cold MAC

- Transfer all aliquots to one 50 ml Falcon tube

- Spin down as above

- Resuspend pellet in 2 ml ice cold MAC

- Transfer to 2 ml RNAse-free microcentrifuge tube

- Spin down at max speed, 4 °C, 10 min

- Decant supernatant

- Shock-freeze pellet in liquid nitrogen

- Store at -80 °C

- FedEx on dry ice