SysMetEx - Standard Operating Procedures

RNA/Protein Samples for leaching experiments(LNU)

Materials:

- 50 ml Falcon tubes

- 15 ml Falcon tubes

- 2 ml RNAse-free microcentrifuge tubes

- Ice cold Mackinthosh medium (MAC)

- Liquid Nitrogen

Procedure:

- Prepare appropriate number of 50 ml Falcon tube containing 25 ml ice cold MAC

- Remove flask from shaker and let mineral settle down for 2 minutes

- Transfer 75 ml supernatant to the tubes containing ice cold MAC (25 ml each)

- Spin down at 14 000 × g, 4 °C, 20 min, slow deceleration

- Resuspend mineral sediment in 40 ml ice cold MAC and transfer to 50 ml falcon tube

- Let mineral settle for 2 minutes and decant supernatant

- Resuspend mineral in 10 ml ice cold MAC and transfer to 15 ml falcon tube

- Let mineral settle for 2 minutes and decant supernatant

- Remove all liquid medium

- Flash freeze mineral pellet in liquid nitrogen

- Take tubes out of centrifuge and decant supernatant

- Resuspend cell pellet in 10 ml ice cold MAC

- Transfer all aliquots to one 50 ml Falcon tube

- Spin down as above

- Resuspend pellet in 2 ml ice cold MAC

- Transfer to 2 ml RNAse-free microcentrifuge tube

- Spin down at 16 000 × g, 4 °C, 10 min

- Decant supernatant

- Flash-freeze pellet in liquid nitrogen

- Store all pellets at -80 °C

- FedEx on dry ice